- Technical Sheet

Click here to email us, or call (972) 961-4655 to speak to a sales representative.

# Hospira/Abbott Plum XL3 Infusion Pump Refurbished, 1 year warranty PRODUCT SPECIFICATIONS. Dimensions:; 13.75"H x 12.2"W x 7.5"D. Weight:; 20 lbs. with batteries. Casing:; High Impact Plastic. Delivery Rate Range:; 0.1-99.9 ml/hr in 0.1 ml increments 100-999 ml/hr in 1 ml increments. Volume to be Infused Range:; 0.1-99.9 ml in 0.1 ml increments 100-9999 ml in 1 ml increments. KVO Rate:; The lower of 1 ml/hr or the primary delivery rate. Occlusion Range:; 10 psig (+5, -2 psig). Power Requirements:; 100-130 VAC, 47/63 Hz, less than 60W. Power Cord:; Hospital grade AC cord, 10ft long. Fuses:; 1.0 A, 250V, slow blowing. Batteries:; Rechargeable and sealed, lead acid 8V batteries. Battery Life:; A fully charged battery set will provide operating time of approximately 4 hours when all pumping units operate simultaneously at a rate of 125 ml/hr or a cumulative delivery of 1000 ml at any combination of rates and pumping units.. Battery Recharge:; Recharge takes approximately 6 hours when the device is turned off..