- Technical Sheet

Click here to email us, or call (972) 961-4655 to speak to a sales representative.

# Alaris/Cardinal IVAC Signature Gold 7130/7230 Infusion Pump Refurbished, 1 year warranty PRODUCT SPECIFICATIONS. Flow Rate Range:; 0.1 to 999.9 ml/hr in 0.1 ml/hr increments (primary) 0.1 to 270.0 ml/hr in 0.1 ml/hr increments (secondary). Volume To Be Infused Range:; .1 to 9999.9 ml in 0.1 ml increments (primary) .1 to 999.9 ml in 0.1 ml increments (secondary). Volume Infused Range:; 0.0 to 9999.9 ml in 0.1 ml increments. KVO Flow Rate:; 0.1-20.0 ml/hr in 0.1 ml/hr increments. Alarms:; Air in Line Battery Depleted Channel Malfunction Computer Link Failure Flow Sensor Unplugged Hold Time Exceeded Instrument Malfunction Key stuck Latch Open No Upstream Flow Detected Occlusion Downstream Occlusion Upstream Primary Flow Detected during. Secondary:; Set Out Set Up Time Exceeded. Dimensions. 7130:; Width 7.7 in/19.3 cm Height 8.6 in/21.8 cm Depth* 5.0 in/12.7 cm Weight 6.85 lbs./3.11 kg Power Cord 10 ft/3 m. 7230:; Width 10.5 in/26.7 cm Height 8.6 in/21.8 cm Depth* 5.0 in/12.7 cm Weight 8.4 lbs./3.7 kg Power Cord 10 ft/3m. Case:; Impact resistant plastic. Power Requirements:; 100- 120 V~,50/60 Hz, 0.5A,3-wire grounded system (40 watts Max). Ground Current Leakage:; Tested to UL Standard 544 and CSA C22.2 No. 125 for medical and dental equipment.. Battery:; Rechargeable nickel cadmium. With a new, fully charged battery, the pump will operate for 4 hours nominal at 100 ml/hr for a two channel instrument operating on both channels simultaneously.. Environmental Conditions. Operating:; Temperature Range 10ºC to 40ºC (50ºF to 104ºF) Relative Humidity 15 to 95% Non-condensing Atmospheric Pressure 632 to 1031 mbar. Storage:; Temperature Range -40ºC to 60ºC (-40ºF to 122ºF) Relative Humidity 5 to 95% Atmospheric Pressure 632 to 1031 mbar.