- Technical Sheet

Click here to email us, or call (972) 961-4655 to speak to a sales representative.

# Baxter APII Ambulatory Pump # Refurbished, 1 year warranty SPECIFICATIONS. Modes of Operation:; PCA, Basal + PCA, and Continuous. Flow Rate Accuracy:; ±10%. Flow Rates:; Basal Rate: 0.1 - 9.9 ml/hr Continuous Rate: 0.1 - 19.9 ml/hr PCA Dose, Bolus, and Priming Rate: 90 ml/hr. Occlusion Detection:; 22 ± 10 psi measured immediately distal of the pumping mechanism. PCA Dose Volume Selection:; 0.0 - 9.9 ml. Bolus Volume Selections:; 0.0 - 9.9 ml. Reservior Volume Selections:; 1 - 1999 ml. One Hour Limit Selections:; 0.1 - 60.0 ml/hr. Delay Time Selections:; 3 - 240 minutes. Power Source:; 9-Volt Alkaline Battery or AC Power Supply. Drive Mechanism:; DC Motor, Microprocessor Controlled, Precision Linear Peristaltic Pumping Mechanism. Housing:; Shock and Vibration Resistant ABS. Dimensions:; 4.9" H x 3.4" W x 1.8" D without Fluid Bag Cover. Weight:; Approximately 1 pound.