- Technical Sheet

Click here to email us, or call (972) 961-4655 to speak to a sales representative.

# Sigma 6000+ Infusion Pump Refurbished, 1 year warranty PRODUCT SPECIFICATIONS. Operating Principle:; Linear peristaltic. Dimensions:; 7-5/8" H x 3-5/8" W x 7-1/2" D (193 mm x 92 mm x 190 mm). Weight:; 10.2 lbs (4.6 kg). Power Requirements:; 105 - 135 volts AC, 50-60 Hz, 0.5 Amps.. Battery Operation:; Approx. 8 hrs - on fully charged battery (18 hr. charge time required). Grounding Resistance:; <0.1 ohm. Electrical Leakage:; <10 microamp. Operating Range:; 0.1-999 ml/hr. Accuracy:; ± 5%. Volume Limit Range:; 0.1-9999 ml. KVO Rate:; 1 ml/hr, <1 ml/hr = infusion rate. Occlusion Pressure:; 12 ±6 psi. Air in Line Detection:; Air bubble in. (0.1 ml) or larger. Alarms. Air:; Proximal Occlusion/Empty Bottle (requires optional flow sensor) Program Complete Malfunctions: H/L Rate, Pump Fail, Multifail and Bad Prom. Occlusion:; Check Tube Open Door Infusion Complete Low Battery/Plug In.