- Technical Sheet

Click here to email us, or call (972) 961-4655 to speak to a sales representative.

# Sigma 8000 Infusion Pump # Refurbished, 1 year warranty PRODUCT SPECIFICATIONS. Flow Range:; 0.1 - 999 mL/hr in 0.1 mL increments from 0.1-99.9 and 1 mL increments above. Accuracy:; +/- 5%. VTBI:; 0.1 - 9999 mL. Total Volume Infused Display:; 0.1 - 9999 mL. KVO Rate:; 0.1 mL/hr. Pump Mechanism:; Linear Peristaltic. Set Type:; Any Standard Gravity IV Set (Abbott, Baxter, B.Braun,and others). Free-Flow Protection:; Yes. Nurse Call Jack:; Optional. MRI Compatible:; No. Data Port:; RS232. Fluid Resistant:; Drip Proof. Power Source:; 105/135 VAC fused T500Ma. 220/240 VAC fused T250Ma. Frequency:; 50/60 Hz. Battery Life:; 5 hours (100 mL/hr). Battery Recharge Time:; 8 hours. Alarms:; Air-in-line Upstream and Downstream Occlusion Improper IV Set Loading Infusion Complete Low Battery Plug in Pump Malfunction Close Roller Clamp Empty Container (with optional flow sensor).